# Hardware for Software Engineers

## Part 1: Logic and "The Glue"

As a software engineer, you are used to writing algorithms that execute sequentially, instruction by instruction, managed by a CPU clock. In hardware engineering, standard logic chips do not wait for a clock cycle. They are like pure, continuously running mathematical functions. If you change the voltage on an input pin, the output pin changes almost instantly.

Here is the translation of the foundational hardware components into software concepts.

### 1. The 74-Series Logic Chips (The Standard Library)

The "74-series" (like the 74HC08 or 74HC32) is a legendary family of microchips. You can think of them as the physical standard library of bitwise operators.

These chips contain physical transistors wired together to form Logic Gates (AND, OR, NOT, XOR).

- 74HC08 (Quad 2-Input AND Gate): This chip has 14 pins. It contains four separate physical AND functions. If you apply 5 volts (a binary 1) to Input A and Input B, the Output pin outputs 5 volts (1). If either input is 0 volts, the output is 0.

- Software Equivalent: return (A && B);

In the 1970s and early 80s arcade boards, if a hardware engineer needed to check if multiple conditions were true at the same time (e.g., "Is the X coordinate at 255 AND is the Y coordinate at 240?"), they had to physically wire up dozens of these small 74-series chips to create a massive, physical IF statement.

### 2. The GAL22V10 (The Programmable Glue)

This is the most critical chip to understand for your Retro2-A design. GAL stands for "Generic Array Logic." If the 74-series chips are a standard library of basic operators, the GAL is a custom, compiled function.

A GAL22V10 has 22 total pins that can be used for logic, with up to 10 of them assignable as outputs. Inside the chip is a massive, unprogrammed grid of AND and OR gates connected by microscopic fuses.

- How it works: You write a text file on your PC using a Hardware Description Language (like CUPL). You define pin names and write boolean equations.

- Example Code: ROM_ENABLE = ADDRESS_15 & !ADDRESS_14;

- The Compiler: You compile this text file into a binary .jed file. You use a USB hardware programmer to flash this file onto the physical GAL chip. The programmer literally burns away specific microscopic fuses inside the chip to permanently wire the internal logic gates to match your code.

- Why we use them: One single 24-pin GAL22V10 can replace about 20 individual 74-series chips. It "glues" the system together. Instead of a messy jungle of wires to create your PPU's video timing, you just write the state machine in code, flash it to the GAL, and drop it on the board.

### 3. How the 6502 and the GAL Work Together (Address Decoding)

The W65C02S CPU has 16 "Address Pins" (A0 through A15). It uses these to shout out a 16-bit binary number representing the memory address it wants to read from or write to.

However, the CPU does not know if an address belongs to the ROM, the RAM, or the Audio chip. It just shouts the number into the void of the copper traces. We must use a GAL to act as a Router or a switch statement. This is called Address Decoding.

- We wire the top address pins of the 6502 directly into the input pins of a GAL22V10.

- We wire the output pins of the GAL to the "Chip Select" (CS) pins of the RAM, ROM, and Audio chips. (A chip ignores the data bus entirely until its CS pin receives a signal).

- We flash the GAL with logic equations:

  - If the address starts with binary 1 (0x8000 to 0xFFFF), pull the ROM's Chip Select pin low.

  - If the address starts with binary 000 (0x0000 to 0x1FFF), pull the RAM's Chip Select pin low.

The GAL sits between the CPU and the memory, continuously evaluating the address lines and waking up the correct hardware at the speed of light.

### Recommended Learning Resources

Before we dive into multiplexers and memory interleaving, here are the absolute best resources to bridge your software knowledge into hardware:

- Ben Eater's "Build an 8-bit computer from scratch" (YouTube): This is mandatory viewing. He builds an entire computer on breadboards using only standard 74-series logic chips, explaining exactly how voltage turns into binary addition, memory, and CPU instructions.

- Ben Eater's "Build a 6502 computer" (YouTube): A follow-up series where he wires a physical W65C02S CPU to ROM and RAM, showing exactly how Address Decoding works with physical wires.

- Nand2Tetris (Course/Book): A fantastic course that starts with a single NAND logic gate and teaches you how to combine them to build a CPU, an assembler, a compiler, and eventually a video game.

## Part 2: Memory, Multiplexers, and Race Conditions

As a software engineer, you interact with memory as a perfectly abstracted concept—you allocate an array, pass a pointer, and assign a value. In hardware, memory is a shared physical space connected by literal copper traces. Managing who gets to talk on those traces without causing a catastrophic collision requires specific routing chips.

Here is how hardware handles pointers, arrays, and thread locking.

### 1. SRAM (The Physical Array)

In software, you declare an array like this: uint8_t vram[32768];

To interact with it, you provide an index and optionally an assignment operator: vram[500] = 0xFF;

A standard single-port Static RAM (SRAM) chip, like the AS6C62256 we spec'd for the system, is the physical embodiment of that exact concept.

- Address Pins (A0-A14): This is your array index. The processor applies binary voltages to these 15 pins to spell out a number between 0 and 32,767. This selects the specific memory slot.

- Data Pins (D0-D7): This is the value inside the array. You either read the voltages on these 8 pins, or you force your own voltages onto them to overwrite the slot.

- Write Enable (WE) Pin: This is the = operator. If this pin receives 5 volts, the chip simply outputs the current data. If this pin is pulled down to 0 volts, the chip saves whatever data is currently on the Data Pins into the selected Address slot.

### 2. The Hardware Race Condition (Bus Contention)

In multithreaded software, if Thread A and Thread B try to write to vram[500] at the same exact moment without a Mutex lock, you get corrupted data or an application crash.

In hardware, this is called Bus Contention, and it is physical. If the W65C02S CPU tries to output 5V (a binary 1) onto a data wire, and the Video PPU tries to output 0V (a binary 0) onto that exact same copper wire simultaneously, you have created a short circuit. Massive current flows between the two chips, data is instantly corrupted, and if left unchecked, the ICs will physically overheat and burn out. We must build a hardware thread lock.

### 3. 74HC157 Multiplexers (The Ternary Operator)

A multiplexer (Mux) is a high-speed routing switch. In code, it is the exact equivalent of a ternary operator or a fast if/else block:

active_address = (clock_is_high) ? cpu_address : ppu_address;

- How it works: A 74HC157 chip has two sets of input pins (Set A and Set B), one set of output pins, and a single "Select" pin.

- If the Select pin is at 0V, the chip physically connects Set A to the outputs.

- If the Select pin is at 5V, it immediately physically connects Set B to the outputs.

By placing a wall of multiplexers right in front of the SRAM's Address pins, we can securely toggle whether the CPU or the Video hardware gets to dictate the array index.

### 4. 74HC245 Transceivers (The Mutex Lock / Floodgate)

While the Mux safely routes the Address (the index), we also need to protect the Data lines (the value) from bus contention. This is where the 74HC245 Octal Bus Transceiver comes in. It is an 8-bit bidirectional buffer that acts as a physical Mutex lock.

- The Magic of "High-Z": Software booleans only have two states: True or False. Hardware pins have a secret third state called High Impedance (High-Z). When a chip enters High-Z, it completely disconnects itself internally from the circuit. It is neither outputting a 1 nor a 0; it is effectively floating in the air, pretending it does not exist.

- By placing a 74HC245 between the CPU's data bus and the SRAM's data bus, we can isolate them. When the Video PPU needs to read memory to draw a tile, we disable the transceiver. The CPU's data pins enter High-Z, physically disconnecting the CPU from the memory bus so the Video hardware can read safely without causing a short circuit.

### 5. Memory Interleaving (Context Switching at 8 MHz)

Now we combine these chips to create the Interleaved Memory architecture that solves the VBLANK bottleneck.

- The W65C02S has a clock signal that acts like a heartbeat. Phase 1 is the low half of the tick, and Phase 2 is the high half.

- We wire that clock signal directly to the Select pin of our 74HC157 Multiplexers and the Enable pin of our 74HC245 Transceivers.

- When the Clock is Low: The multiplexers snap into position A, connecting the Video PPU to the SRAM address pins. The transceiver enters High-Z, locking the CPU out. The Video PPU safely reads the graphics data it needs for the CRT monitor.

- When the Clock is High: The multiplexers instantly snap to position B, connecting the CPU to the SRAM address pins. The transceiver opens the data floodgates. The CPU seamlessly executes its game logic and writes to memory.

This physical "context switch" happens 8 million times a second. To your C game engine, it feels like you have 100% exclusive, uninterrupted access to the memory array, but the hardware is secretly trading ownership of the array back and forth on every microscopic tick of the clock.

## Part 3: Variables, APIs, and the Infinite Loop

In software, when you want to save a value, you declare a variable. When you want systems to talk, you call an API. When you want to repeat an action, you write a for loop. Hardware has direct, physical equivalents to all of these concepts, and they are the building blocks of your custom Video PPU.

### 1. The Latch (The Hardware Variable)

In C, if you write uint8_t scroll_x = 120;, the compiler assigns a memory address to hold that value. But what if a standalone hardware circuit (like your video generator) needs its own local variable that the CPU can update? We use a Latch (specifically, the 74HC573 Octal Transparent Latch).

- How it works: A 74HC573 has 8 input pins, 8 output pins, and a "Latch Enable" (LE) pin.

- If the LE pin is high (5V), the outputs directly mirror the inputs.

- The magic: When the LE pin drops to 0V, the chip takes a snapshot of whatever voltages were on the input pins at that exact millisecond. It "latches" them. Even if the input wires completely change or disconnect, the 8 output pins will continuously output that saved snapshot forever (or until LE is pulled high again).

- Software Equivalent: It is a physical, 8-bit local variable. The outputs of this chip feed directly into your video math logic to offset the background.

### 2. Memory-Mapped I/O (The Hardware API)

How does your C game engine actually pass the scroll_x value from the W65C02S CPU into that physical Latch chip on the other side of the board? Through Memory-Mapped I/O.

The W65C02S doesn't have "USB ports" or special API functions to talk to other chips. It only knows how to read and write to memory addresses. So, we trick it.

- The Setup: We assign a fake memory address to the Latch. Let's say 0x2005.

- The Decoupling: We program one of your GAL22V10 chips to constantly monitor the CPU's address lines. We write a rule: If the CPU writes to 0x2005, DO NOT activate the System RAM.

- The API Call: Instead, the GAL instantly routes the CPU's "Write" signal directly to the Latch Enable (LE) pin on the 74HC573.

- The Result: Your C code executes memory[0x2005] = 120;. The CPU thinks it just saved data to RAM. In reality, the GAL intercepted the write, activated the Latch, and the video hardware instantly snapped up the new scroll_x variable. This is exactly how retro consoles handle controller polling, audio generation, and graphics configuration.

### 3. Binary Counters (The Hardware for Loop)

A hardware video generator is essentially two massive, infinitely running for loops: one counting the X pixels across the screen, and one counting the Y scanlines down the screen. Hardware achieves this using a Binary Counter (like the 74HC161).

- How it works: A 74HC161 is a 4-bit counter. If you chain two of them together, you get an 8-bit counter (0 to 255). It has a "Clock" input pin and 8 output pins.

- Every time the Clock pin receives a pulse (a rising edge from 0V to 5V), the chip internally adds 1 to its current value and updates its output pins in pure binary.

- Software Equivalent: for(int x = 0; x < 256; x++) { ... }

- The Video Application: You wire a fast crystal oscillator (your pixel clock) to the X counter. The counter rapidly counts 0, 1, 2, 3... up to 255. These output pins are wired directly to the address pins of your Video RAM. As the counter counts up, it physically steps through the memory array, fetching the next pixel on the scanline at the exact speed the CRT electron beam is moving across the glass tube.

- When the X counter hits 255, a logic gate detects it, resets the X counter to 0, and sends a single clock pulse to the Y counter (incrementing the scanline).

By combining Latches (variables), Memory-Mapped I/O (APIs), and Counters (loops), you have the core infrastructure of an independent video chip.

## Part 4: The Render Pipeline

In modern front-end or full-stack software development, you wouldn't render a massive, uncompressed 1080p bitmap to the DOM if you could just repeat a small, efficient tiled background image. 8-bit hardware enforces this optimization physically. The Render Pipeline is essentially a series of high-speed hardware lookup tables resolving pointers into pixel colors.

### 1. The Nametable (The Pointer Array)

Your interleaved Video RAM (VRAM) does not hold actual colors. It holds a 32x30 grid called the Nametable.

- The Software Concept: It is a 2D array of 8-bit integers. Each byte is simply an index (a pointer) ranging from 0 to 255.

- The Hardware Reality: As the physical X and Y counters (the hardware for loops from Part 3) track the CRT beam across the screen, their top binary bits are wired directly to the VRAM's address pins. This continuously fetches the current tile index (e.g., Tile #42) that the beam is currently hovering over.

### 2. The Pattern Table (The Asset Library)

Your CHR-ROM chip holds the actual pixel art—the raw bitmap data for your 8x8 tiles. In software, finding a specific pixel inside a tile requires a nested array lookup. In hardware, it is solved by physically wiring two numbers together to create a master address.

- The Concatenation: The hardware takes the Tile Index it just fetched from VRAM (e.g., #42) and physically bundles those wires together with the lowest 3 bits of the Y-counter (which represent the specific row, 0-7, inside the 8x8 tile).

- The Lookup: This combined bundle of wires is plugged directly into the address pins of the CHR-ROM chip.

- The Result: The CHR-ROM instantly outputs the raw binary pattern for that exact row of that exact tile. No math or calculation is required by the CPU; it is a pure hardware memory fetch occurring in nanoseconds.

### 3. Attributes and Palettes (Hardware CSS)

To save memory, a raw tile pattern only dictates which pixels are "on" or "off." To apply color, the hardware uses a separate VRAM array called the Attribute Table.

- The Software Concept: Think of this as applying a CSS class to a div. You are telling the hardware, "Apply Palette #3 to this specific 2x2 grid of tiles."

- The Hardware Reality: As the tile pattern is being fetched, another logic circuit fetches the attribute byte. A hardware multiplexer (the ternary operator switch from Part 2) intercepts the raw pattern data and instantly translates it into a final 8-bit color index based on the chosen palette.

### 4. The Sprite Compositor (Hardware Z-Index)

While the background pipeline is endlessly looping, a completely separate hardware system manages the sprites.

- The Evaluation Phase: Sprites live in your Object Attribute Memory (OAM) array, structured strictly as sequential Y, Tile, Attribute, X groups. During the horizontal blanking period of every scanline, a hardware math circuit (an Adder/Subtractor) rapidly scans all 64 Y-coordinates in OAM.

- The Line Buffer: If a sprite's Y-coordinate falls on the upcoming scanline, the hardware fetches its pattern from the CHR-ROM and saves the row of pixels into a small temporary memory chip called a shift register. It will only do this for a maximum of 16 sprites before it stops scanning (the hardware sprite limit).

### 5. The Final Multiplexer (The Pixel Collision)

As the CRT beam moves across the screen, the hardware is now holding two pieces of data at once: the Background pixel and the Sprite pixel.

- The Decision: All of this data flows into one final, high-speed multiplexer right before the Video DAC.

- The Logic: The hardware checks the sprite pixel. If the sprite pixel is "transparent" (usually color index 0), the multiplexer stays in position A, sending the Background pixel to the screen. If the sprite pixel has a color, the multiplexer violently flips to position B, physically blocking the background signal and sending the Sprite pixel to the DAC instead.

This entire five-step pipeline happens completely independently of the W65C02S CPU. The CPU's only job is to update the VRAM Nametables and the OAM array during the interleaved clock cycles; the hardware pipeline handles the rest automatically, millions of times per second.

## Part 5: Timing, Audio, and the Retro2-A Loop

In standard 8-bit consoles, system timing is a frantic, cycle-counted nightmare. Because of the hardware choices you made for the Retro2-A arcade board, your system timing actually behaves much more like a modern, decoupled microservice architecture.

### 1. The "Dead" NMI (How Interleaving Changes Everything)

In a stock NES, the Non-Maskable Interrupt (NMI) triggered by the screen's Vertical Blanking (VBLANK) period is a terrifying deadline. The CPU drops everything and has about a millisecond to furiously push sprite data into VRAM before the screen locks it out again.

Your Architecture: Because you chose the interleaved memory model, your W65C02S CPU has 100% uninterrupted access to VRAM. It never gets locked out.

- Therefore, your VBLANK NMI is no longer a frantic data-transfer deadline. It simply becomes a 60Hz Metronome.

- When the GAL detects the screen is done drawing, it fires the NMI to the 6502. Your C engine's interrupt handler simply updates a boolean: frame_ready = true;. That's it. It allows your game loop to run at a perfectly stable 60 FPS without ever worrying about screen tearing or locked memory.

### 2. The ATmega Coprocessor (The Hardware Microservice)

Generating audio requires constant, brutal math. If the W65C02S had to manually push the speaker cone in and out hundreds of times a second to make a square wave, it would have zero time left to calculate your AABB collisions.

We solve this using the ATmega microcontroller as an Audio Coprocessor.

- The Software Concept: This is exactly like offloading a heavy database query to a background Web Worker so the main UI thread doesn't freeze.

- The Hardware Reality: The ATmega is a completely independent computer sitting on the board, running its own C++ loop. It uses its internal hardware timers to generate the pulse, triangle, and noise waves.

- The Bridge: We map the ATmega to a specific memory address (e.g., 0x4000) on the W65C02S. When your game wants the player to jump, your W65C02S simply executes memory[0x4000] = JUMP_SOUND_CMD;. The ATmega catches that byte, instantly starts synthesizing the sound, and the W65C02S immediately goes back to calculating physics.

### 3. The Retro2-A Main Loop

When you put the interleaved memory, the hardware variables (Latches), and the ATmega coprocessor together, the C engine for your arcade board ends up looking remarkably clean. It completely avoids the spaghetti assembly code usually required for retro development.

Here is the pseudo-C structure of how your specific hardware actually runs:

volatile bool frame_ready = false;

// The Hardware Interrupt (Fires 60 times a second)

void nmi_handler() {

frame_ready = true;

}

void main() {

init_system();

while(1) {

// Wait for the hardware 60Hz metronome

if (frame_ready) {

frame_ready = false;

// 1. Read Inputs (Memory-mapped 74HC165 shift registers)

uint8_t p1_input = read_controllers();

// 2. Execute Game Logic (100% CPU time available)

calculate_aabb_collisions();

update_player_state(p1_input);

// 3. Update VRAM instantly (Thanks to Interleaved Memory)

update_oam_array();

update_nametable_seams();

// 4. Trigger Audio (Fire-and-forget to the ATmega)

if (player_jumped) {

audio_register = CMD_PLAY_JUMP;

}

}

}

}

This architecture completely shields you as a software engineer from the underlying electrical timing, allowing you to write clean, logical C code that outputs pure 8-bit analog signals.