# Retr01 Low-Level Emulator Specification

## 1. Objective

To bridge the gap between software development and the physical discrete-logic motherboard, a bespoke, hardware-restricted emulator must be built. This emulator will serve as the testing ground for binaries compiled via PPUX and cc65/Rust. Its primary goal is not speed, but cycle-accurate architectural validation to ensure zero divergence when the ROM is flashed to the physical Retr01 board.

## 2. Core Components to Emulate

### The CPU Core

The emulator will integrate a standard, cycle-accurate 6502 software core (adapted for the W65C02S instruction set). It must strictly observe the two-phase clock cycle (Phase 1 Low / Phase 2 High) to properly simulate memory bus ownership.

### The Interleaved Memory Subsystem

Traditional emulators give the CPU and PPU simultaneous access to memory blocks. The Retr01 emulator must enforce the physical interleaved bus logic:

- During Clock Phase 1: The CPU is locked out of VRAM. The emulator's Virtual PPU executes its background and sprite fetch cycles.

- During Clock Phase 2: The Virtual PPU is locked out. The 6502 core is granted access to read/write to the AS6C62256 VRAM arrays.

### Address Decoding (The Virtual GALs)

Instead of hardcoding memory maps in software, the emulator should implement a "router" function that acts as the GAL22V10 chips. It reads the 16-bit address from the 6502 and routes the read/write calls to the appropriate arrays (System RAM, VRAM, OAM, or the ATmega APU memory-mapped registers).

## 3. The Virtual PPU (Graphics Rendering)

The rendering pipeline must strictly enforce the physical limitations of the Retr01 design:

- 2-Bit Color Depth: The pipeline must read CHR-ROM data exactly as 2bpp planes, resolving to 3 colors plus transparency via the palette attributes.

- The 16-Sprite Limit: During the horizontal blanking period emulation, the code must scan the 64-sprite OAM array. It will populate a virtual line buffer with a maximum of 16 sprites. Any sprites beyond 16 on that scanline must be intentionally dropped (hardware accuracy).

- Multiplexing & Transparency: The pixel compositor must accurately mimic the 74HC157 multiplexer logic, pushing background pixels unless a non-zero (non-transparent) sprite pixel is present in the line buffer.

## 4. Audio Coprocessor Emulation

The emulator must trap memory writes destined for the ATmega coprocessor's address space. It will read the command bytes (e.g., pulse wave frequency, envelope triggers) and synthesize the NES-style audio locally using a standard audio API (like SDL2 Audio or WebAudio) to simulate the physical ATmega's output.

## 5. Development Workflow Integration

The emulator will be designed to instantly reload when a new ROM binary is generated. The workflow is:

- Generate nametables and 2bpp CHR graphics in PPUX.

- Write engine logic and compile via cc65.

- Launch the Retr01 Emulator passing the output .bin file as an argument for immediate, hardware-accurate testing.