# Retr01-A (Arcade Edition) General Concept

## 1. System Overview

The Retr01-A is the arcade-focused foundation of the Retr01 hardware ecosystem. It is a high-performance, 2D arcade-grade motherboard engineered to be physically robust, technically transparent, and easily interfaceable for custom game development.

- Data Paradigm: Strictly binary-oriented, interpreting raw and compressed binary data directly without dynamic memory allocation.

- Execution Model: Built around an interleaved memory architecture, eliminating traditional VBLANK bottlenecks and granting the CPU 100% of the frame to execute game logic.

- Development Pipeline: Designed to execute deterministic ROMs built using the open-source PPUX asset tool and tested on the Retr01 Low-Level Emulator.

## 2. Physical Arcade Hardware

The initial Retr01-A design utilizes standard through-hole (DIP) components. This choice prioritizes ease of prototyping, modular testing, and field repairability.

| Component | Specification |
| --- | --- |
| CPU | Western Design Center W65C02S (DIP-40) |
| Programmable Logic | Optimized GAL22V10 array (DIP-24) driving state machines, timing, and bus arbitration. |
| Memory | Interleaved AS6C62256 (32KB) SRAM, managed by 74HC-series multiplexers and transceivers. |
| Storage | Non-volatile parallel EEPROM (e.g., AT28C64B) for high scores and system settings. |

## 3. Controller & I/O Interface

The Retr01-A utilizes a professional-grade IDC interface for arcade cabinet integration.

- Connector: 40-pin IDC Box Header (Raspberry Pi GPIO style).

- Allocation: 24 pins for player inputs (2 Joysticks, 8 action buttons per player), 16 pins for power, ground, and system signals (Coin, Start, Reset).

## 4. Video & Audio Output

Engineered for authentic retro-arcade signal quality and strict memory aesthetics:

- Graphics Engine: Strict 2bpp color depth, yielding 3 colors plus transparency per tile/sprite to ensure rapid memory fetches and distinct aesthetic styling.

- Video Output: Resistor-ladder DAC generates analog RGBS signals. Includes optional encoder (e.g., AD724/CXA1645) for S-Video/Composite output via solder pads.

- Audio: Dedicated ATmega-based Audio Processing Unit (APU) synthesizing NES-style mono channels via register writes from the W65C02S.

## 5. Cartridge & ROM Architecture

Utilizes 5V Parallel Flash ROM (e.g., AM29F016B) with the following mandatory memory structure:

- PRG-ROM: Game logic instructions compiled via cc65 (up to 512 KB).

- CHR-ROM: Uncompressed 2bpp pattern data (up to 128 KB).

- Map Data: RLE compressed level data (up to 1.17 MB).