# Retr01-C (Console Edition) General Concept

## 1. System Overview

The Retr01-C is the living-room console variant of the Retr01 ecosystem. It is designed to sit under a display, output clean video signals, handle standard controllers, and provide a comfortable, solder-repairable baseline using traditional components before moving to miniaturized designs.

## 2. Board Architecture & Component Layout

Unlike surface-mount designs reserved for portable form factors, the initial Retr01-C build leverages through-hole (DIP) components and socketed ICs. This ensures maximum ease of debugging, breadboard-to-PCB translation, and hardware hacking.

- The Silicon Footprint: Utilizes the standard DIP-40 W65C02S processor alongside DIP-packaged SRAM and standard logic chips, making signal probing with an oscilloscope straightforward.

- Layer Stackup: Designed on a clean 2-layer or standard 4-layer PCB, prioritizing a solid ground return path to keep RF emissions low when plugged into home display units.

- Form Factor: Clean mini-ITX or custom console shell footprint, providing ample edge real estate for physical ports and connectors.

## 3. Video Output & Display Interfacing

Engineered to interface cleanly with modern or legacy home displays while strictly enforcing the 2bpp (3 colors + transparency) limit:

- Digital Video (HDMI / DVI): Programmable logic handles pixel-doubling and nearest-neighbor upscaling, feeding an encoder chip for crisp 720p/1080p output without blurry scaling artifacts.

- Legacy Analog (RGB / Component / S-Video): Resistor-ladder DACs generate standard composite, S-video, or RGB signals routed to a multi-out port for authentic CRT television play.

## 4. Controller Ports & Peripheral Interfacing

- Physical Ports: Integrates classic DB-9 controller ports (compatible with Sega Genesis or Atari-style controllers) directly onto the board edge.

- Zero Latency Polling: Controller states are captured directly at the hardware boundary via programmable logic before the CPU polls the frame, ensuring absolute zero input lag.

## 5. Power Delivery

Leverages stable wall power via a standard barrel jack or modern USB-C Power Delivery (PD), stepping down cleanly to 5V and 3.3V for reliable operation across all logic banks.